How to back up Antlia

Michel Anciaux, 8 August 2001

This describes how to back up the entire Antlia disk and later restore it. This is a cpu intensive task and should not be performed while observing.

# Backing up the disk

Log in on Antlia as michel/haaa.

Type reset.

Wait one minute.

Log in again as michel/haaa.

Type chd /h0.

Type del /h0/sys/backup/h0backup.

Type fsave –f=/h0/sys/backup/h0backup –m=/h0/sys/backup/dates –l=0.

This may take a bit more than one hour to complete.

The complete backup is a file: “h0backup” in “/h0/sys/backup”. This file should be ftp’d to another machine (use binary transfer) and burnt into a CD. Normally I also restore the backup on Fornax. This keeps the two systems synchronised.

# Restoring the backed up disk

Log in on Antlia as michel/haaa.

Type reset.

Wait one minute.

Login again as michel/haaa

Read the “h0backup” file from CD or from another support.

Ftp it to Antlia (or Fornax) in ”/h0” (use binary transfer).

Type copy /h0/h0backup /h0/bu -r.

Type chd /h0.

Type frestore –f=/h0/bu –eb=64 –sqa.

This may take up to two hours.

Note: Since both Antlia and Fornax have a minimal operating system with FTP and TELNET on EPROM, the restore procedure can be used even if the hard disk dies. In that case after installing a new disk, you would have to format the new disk before restoring the backup.

# Caution: use the following with great care!

To format the disk:

Type iniz /h0fmt.

Type format /h0fmt –c=16.

You can now use the normal restore procedure.